Project: MELLA 
Board: ESP32

Device Working & Functional Overview

1. Device Purpose

This device is an intelligent heating controller designed to:

- Safely control a heater using a relay

- Allow users to configure WiFi credentials via a mobile or laptop

- Automatically manage heating based on temperature and knob setting

- Provide clear visual feedback using status LEDs

The device is designed with safety, reliability, and user clarity as top priorities.

2. Power ON / System Boot Behaviour

- As soon as the device is powered ON or rebooted:

  - 🟡 YELLOW LED turns ON

  - This indicates the device is booting and loading its system

During this phase:

- Heater (Relay) is always OFF

- No heating operation is allowed until the system is fully ready

3. WiFi Connection & Provisioning Logic

3.1 Existing WiFi Available

- If WiFi credentials are already stored:

  - The device automatically attempts to connect to the configured WiFi network

  - During this time:

    - 🟡 YELLOW LED blinks slowly (indicating connection attempt)

- If the connection is successful:

  - The system enters working state

  - 🟢 GREEN LED turns ON

  - 🟡 YELLOW LED turns OFF

3.2 WiFi Not Available / First-Time Setup

If:

- No WiFi credentials are stored, or

- The device fails to connect to the saved WiFi

Then:

- The device automatically switches to Setup / AP Mode

- It creates its own WiFi hotspot named:

MELLA-XXXX

(Default password: setup1234)

- In this mode:

  - 🟡 YELLOW LED blinks fast

  - Heater remains OFF for safety

3.3 WiFi Setup via Web Interface

- User connects to the device WiFi (MELLA-XXXX)

- Opens browser and navigates to:

- http://192.168.4.1

- A setup page is shown where the user:

  - Enters WiFi SSID and Password

- Credentials are saved securely in device memory

- Device automatically reboots and connects to the selected WiFi

4. Normal Working State (System Ready)

Once the device is:

- Successfully connected to WiFi

- Fully initialized

The system enters Working State.

Visual indication:

- 🟢 GREEN LED ON

- 🟡 YELLOW LED OFF

This means:

The device is healthy, connected, and ready to operate

5. Heater / Relay Operation Logic

The heater is controlled using:

- A temperature sensor

- A user knob (level selection)

- Internal safety rules

Key safety rule:

The heater can operate ONLY when the system is in Working State

If WiFi is lost or the system is not ready:

- Heater is immediately switched OFF

6. GREEN LED Behaviour (Heater Status)

The GREEN LED also provides heater status feedback:

🟢 GREEN LED – SOLID ON

- System is working

- Heater is currently OFF

- Temperature has reached or exceeded the required threshold

🟢 GREEN LED – BLINKING

- System is working

- Heater is currently ON

- Active heating is in progress

This allows the user to visually confirm:

- Whether the device is alive

- Whether heating is currently happening

7. Temperature Control Logic

- User sets desired heating level using the knob

- Each knob level corresponds to a predefined temperature threshold

- Device continuously:

  - Reads temperature sensor

  - Compares actual temperature with threshold

- Heater logic:

  - If temperature is below threshold → Relay ON (Heating)

  - If temperature is above threshold → Relay OFF

8. Safety Behaviour (Critical)

The device includes strict safety enforcement:

- Heater is always OFF when:

  - Device is booting

  - WiFi is not connected

  - Device is in AP / setup mode

  - Factory reset is in progress

This ensures:

- No unintended heating

- Safe operation even during failures or resets

9. Factory Reset Functionality

- Device includes a physical reset button

- Long press triggers factory reset:

  - WiFi credentials are erased

  - Device ID remains preserved

  - Heater is immediately switched OFF

  - Visual feedback is shown using RED LED

- Device reboots into setup mode after reset

10. LED Status Summary (Quick Reference)

| Condition | YELLOW LED | GREEN LED | Heater |
| --- | --- | --- | --- |
| Booting | ON | OFF | OFF |
| WiFi connecting | Slow blink | OFF | OFF |
| AP / Setup mode | Fast blink | OFF | OFF |
| System ready, heater OFF | OFF | ON (solid) | OFF |
| System ready, heater ON | OFF | Blinking | ON |
| Reset / fault | RED indication | OFF | OFF |

11. Overall System Assurance

This device is designed to:

- Be safe by default

- Prevent heating during unstable states

- Clearly communicate system and heater status to the user

- Automatically recover from WiFi failures

- Provide an easy and guided setup experience