Please find below the clean, client-friendly, professionally structured document that clearly shows the major and minor changes required to migrate from ESP-12F (ESP8266) to ESP32-C5.

This will help all the stake holders to understand that this is NOT a “straightforward swap,” but a full platform migration that affects firmware, hardware, and even the mobile provisioning process.

Migration Document

Migration of Existing Firmware from ESP-12F (ESP8266) to ESP32-C5

(Detailed Major & Minor Changes Required)

1. Hardware-Level Changes

1.1 Major Changes

| Area | ESP-12F (ESP8266) | ESP32-C5 | Impact |
| --- | --- | --- | --- |
| Processor Architecture | Single-core | Dual-core (WiFi + Task Core) | Requires firmware task restructuring |
| Flash Interface Pins | GPIO9/10 unusable | Different flash pin mapping | PCB re-routing required |
| Boot/Strap Pins | GPIO0, GPIO2, GPIO15 critical | Different strap pins | PCB redesign – cannot reuse old routing |
| ADC Behavior | Only 1 ADC pin (A0), 0–1.0 V | Multiple ADC pins, 0–3.3 V (with attenuation) | Potentiometer circuit must be redesigned |
| Power Requirements | ~200–250 mA peak | Up to ~500–600 mA peak | Must redesign power supply section. The Old Power Controller will not work. |

1.2 Minor Changes

- Status LED pin likely changes.

- Relay control pin may need change for safe boot.

- DS18B20 sensor pin must map to allowed ESP32 IOs.

- Board must expose EN & BOOT pins differently for programming.

2. Firmware-Level Changes

2.1 Major Changes

A. WiFi Library & Core Networking

| Functionality | ESP8266 API | ESP32 API | Impact |
| --- | --- | --- | --- |
| WiFi | <ESP8266WiFi.h> | <WiFi.h> | Direct replacements required |
| Multi-AP | ESP8266WiFiMulti | WiFiMulti | Code rewrite needed |
| SmartConfig | Uses ESPTouch 1.0 | Uses ESPTouch 2.0 | Mobile app compatibility breaks. |

B. EEPROM & Flash Memory

- ESP8266 uses EEPROM emulation (512 bytes)

- ESP32 uses:

  - Preferences (NVS) — recommended

  - or optional EEPROM emulation (Currently we are using EEPROM)

➡ All EEPROM read/write logic must be rewritten
➡ CRC storage structure must be re-designed

C. ADC / Knob Reading

ESP8266:

analogRead(A0) // 0–1023 (1.0V)

ESP32:

- analogRead(GPIOxx) → 0–4095, multiple ADC units, required attenuation config

- Voltage range is different

- Scaling must be rewritten

➡ Entire potentiometer reading logic must be revised.

D. GPIO Pin Behavior

- ESP8266 pins can be used freely except 0, 2, 15

- ESP32-C5 has:

  - Strapping pins

  - Restricted pins

  - DAC/ADC-only pins

  - Touch pins

➡ Every pin used (Relay, Knob, DS18B20, LED) must be reassigned and revalidated.

E. Timers / Ticker

- ESP8266 Ticker runs on simple interrupt.

- ESP32 uses FreeRTOS timers and different clock behavior.

➡ All scheduling/timing logic must be re-tested.

F. OTA / HTTP Update

- ESP8266HTTPUpdate → not available on ESP32

- Must use HTTPUpdate or Update class

- Firmware update URL logic must be re-written

➡ OTA update code must be migrated and revalidated.

G. SmartConfig (Provisioning)

This is a critical point for this project:

SmartConfig on ESP8266 ≠ SmartConfig on ESP32

| Platform | SmartConfig Version | App Compatibility |
| --- | --- | --- |
| ESP8266 | ESPTouch v1 | Older ESPTouch apps |
| ESP32 | ESPTouch v2 | Requires ESPTouch v2-compatible mobile app |

The existing mobile app will NOT work without modification.
The provisioning protocol changed, requiring app update.

➡ This is a major mobile-side impact.

2.2 Minor Firmware Changes

The internal code/syntax must be changed to work efficiently.

3. Application Flow Impacts

3.1 WiFi Provisioning (Critical)

The mobile app originally uses:

- SmartConfig (ESPTouch v1)

ESP32 uses:

- ESPTouch v2 (different packet structure & encryption)

➡ Mobile app must be updated

- New library

- New provisioning method

- New MDNS/discovery if used

- New UI workflow (if BLE is added later)

4. Testing & Validation Requirements

Major Testing Areas

- Power supply stability

- ADC calibration

- Temperature accuracy

- Relay switching

- EEPROM/NVS read/write

- Provisioning success rate

- MQTT connectivity

- OTA firmware update

- Boot reliability from cold start

Minor Testing Areas

- UI timing

- LED behavior

- WiFi reconnect logic

- Sensor error handling

5. Summary (High-Level Bullet List)

Here is the executive summary version:

Migrating ESP-12F → ESP32-C5 is NOT a simple chip swap

It requires:

Hardware Changes

- Completely new PCB routing

- New boot strap layout

- New ADC input design

- New power supply section

- New GPIO mapping

Firmware Changes

- WiFi library replaced

- SmartConfig provisioning logic rewritten

- EEPROM storage replaced

- ADC reading rewritten

- Timer/interrupt logic updated

- OTA update code changed

- All GPIO pin behavior revalidated

Mobile App Changes

- Current app using old SmartConfig will NOT work

- Must update provisioning library

- Must re-test UX, error handling

- Or replace with SoftAP or BLE provisioning