I want to create a mobile application that should have the following set of features. This should connect with the IoT device and helps the user to connect with the APMode Wifi, and further helps to store the WiFi username and password to the EEPRom of the ESP32 module.

- The Application will have one Button called “Add Device”

- After clicking on that Button, the System will Populate the List of all the WiFi Adaper name that starts with the name “Amazon”.

- The User will select that Adapter i.e. that starts with “Amazon”.

- This Amazon name will be the WiFi APMode of my ESP32 Module

- As soon as the User Selects the Amazon named WiFi, the Mobile will connect with that Adapter and connects with the 192.168.4.1 IP and give an Option to enter the WiFi name and the Password that the User would like to store in the ESP32 Module for it to use for the Future. The ESP code is using “POST” method to call “save” function.

- The ESP32 Module has the “attached” code to get the WiFi, where-as the Mobile application should create its own TextBox for WiFi name and Password.

- The User enters the Username and password, clicks on the “Continue” Button, the Application stores the Username/Password in the EEPRom on ESP32, the ESP32 Module starts its own logic, where-as the Module application, Stores the Value of the Amazon name in the Mobile application.

- The following is the code that has been written in the ESP Module.

//CONFIG PORTAL

void startConfigPortal() {

WiFi.mode(WIFI_AP);

WiFi.softAP(defaultAP_SSID.c_str(), defaultAP_PASS);

IPAddress apIP = WiFi.softAPIP();

Serial.print("Config AP IP: ");

Serial.println(apIP);

showConfigMode();

server.on("/", []() {

String page =

"<!DOCTYPE html>"

"<html>"

"<head>"

"<meta charset='utf-8'>"

"<title>AmazonBox Wi-Fi Setup</title>"

"</head>"

"<body>"

"<h2>AmazonBox - Wi-Fi Configuration</h2>"

"<p>Enter your Wi-Fi details below:</p>"

"<form action='/save' method='post'>"

"SSID:<br>"

"<input type='text' name='ssid'><br><br>"

"Password:<br>"

"<input type='password' name='pass'><br><br>"

"<input type='submit' value='Save & Reboot'>"

"</form>"

"<br><br>"

"<p>If you want to clear saved credentials, visit "

"<a href='/clear'>/clear</a></p>"

"</body>"

"</html>";

server.send(200, "text/html", page);

});

server.on("/save", HTTP_POST, []() {

String ssid = server.arg("ssid");

String pass = server.arg("pass");

saveCredentialsToEEPROM(ssid.c_str(), pass.c_str());

server.send(200, "text/html", "Saved! Rebooting...");

delay(2000);

ESP.restart();

});

server.begin();

while (true) {

server.handleClient();

delay(100);

}

}