- Complete Functional Flow — IoT Wax Machine (ESP32-C5)

- 1 — High level sequence (one-line)

- Power ON → Self-test & First-time check → Wi-Fi Provisioning (if needed) → Cloud subscription check → Normal operation (Manual or App control) → Temperature control loop (DS18B20 + Knob) → Safety & Overtemp handling → OTA / Diagnostics → Power OFF / Safe shutdown.

- 2 — Detailed step-by-step functional flow

- A. Power ON sequence

- User supplies AC mains power to the Power PCB.

- Power PCB performs input surge protection and generates regulated 5V rail.

- Logic PCB (ESP32-C5) receives 5V, MCU boots.

- Bootloader runs — check boot counters, last OTA status.

  - If previous OTA failed and rollback available → rollback to last stable partition and boot.

- Self-test (0–3 seconds): read DS18B20 presence, ADC for knob, check relay driver MOSFETs, LED driver, and internal flash health.

  - LED: Blink white during self-test.

- First-time / Factory initialization check:

  - MCU reads persistent flags in NVS (non-volatile storage).

  - If factory_new == true or not configured:

    - Enter Configuration Mode (see provisioning).

  - Else proceed to Network Connect.

- B. Configuration / Provisioning Mode (when device is unconfigured or user requests reset)

- Device advertises temporary Wi-Fi AP with unique SSID: WAX-<SHORT-UID> (e.g., WAX-AB12C3).

  - SSID composition: prefix + unique product ID (client to confirm format).

- LED: Orange blinking (slow) — indicates configuration AP active.

- Mobile App flow:

  - User logs into app with their username/password (per client backend).

  - App → Add Product → Scan QR or enter UID.

  - App connects to device AP; sends customer Wi-Fi credentials and product auth token (if required).

  - Device authenticates mobile packet (simple shared key or on-the-fly token from backend).

- Device receives Wi-Fi config and attempts to connect to the configured Access Point.

  - LED: change to Orange solid while trying; if fails after N attempts → blink orange fast (indicates provisioning failed).

- If Wi-Fi connect success:

  - Device calls backend subscription API to validate UID & subscription status.

  - If subscription valid → device proceeds to Registered state; LED turns Green solid.

  - If subscription invalid → LED turns red/orange pattern (client specified), device refuses to operate heater (unless client says allow trial mode); mobile app shows error with instructions.

- Timeout & Fallback:

  - If no app connects within X minutes (configurable), device reverts to factory_new or retains AP for a configurable window. Optionally allow WPS or SmartConfig if required.

- C. Normal Operation (Registered & Connected)

- Operational LED: Green solid (connected & ready).

- Local control: User can operate via knob (physical) or mobile app (remote control over local network/cloud).

- Knob behavior:

  - Knob (potentiometer or encoder) maps to setpoint temperature T_set (range defined by product).

  - If user adjusts knob: immediately update local setpoint; send telemetry to backend asynchronously.

  - LED color/animation: when knob moved, LED flashes white momentarily to show change acknowledged, then returns to mode color or indicates heating if element on.

- Temperature control loop:

  - Sensor: DS18B20 supplies T_current every t_sensor seconds.

  - Control algorithm: hysteresis or simple PID:

    - If T_current < T_set − H (hysteresis) → heater ON (relay closure).

    - If T_current ≥ T_set + H → heater OFF.

  - LED:

    - White solid while heating (or alternate: white pulsing to show active heating).

    - Red when over-threshold or fault.

    - Green when stable and within set range.

- Manual override / Buttons:

  - Provide long-press / double-press sequences for factory reset, enter provisioning again, or enable demo mode.

  - LED patterns indicate action success/failure.

- Cloud & App updates:

  - Device sends periodic telemetry: UID, T_current, T_set, heater_status, uptime, firmware_version, RSSI.

  - If app issues direct command (e.g., set temperature or start/stop), server validates subscription & sends command to device via MQTT/HTTP push or device polling. Device acknowledges and acts.

- D. Safety, Over-Temperature & Fault Handling

- Soft threshold & hard limit:

  - T_set is user target. T_soft_limit = T_set + H1. Heater will switch off and alert if T_current > T_soft_limit.

  - T_hard_limit: e.g., sensor detects above safe physical limit (e.g., 120°C) → immediate heater disable latch, log error, LED red flashing, notify cloud. Manual reset required.

- Sensor failure:

  - If DS18B20 disconnected or reading CRC fails → enter Fault state: heater OFF, LED red blink, alert to app/cloud.

- Relay / SSR fault:

  - If expected heater current/feedback not detected over N cycles → fault state, lock heater, LED red.

- Power loss:

  - On mains loss, Power PCB must maintain power long enough for safe shutdown (supercap or small battery) to close logs & gracefully store state. If not available, on next boot check previous state and perform safety checks.

- E. OTA & Firmware Management

- OTA triggers:

  - Manual from App (only authorized users).

  - Scheduled by backend (after notification).

- OTA procedure:

  - Device downloads image to inactive partition and verifies checksum/signature.

  - If verification passes → device switches boot partition on next reboot or uses atomic swap mechanism.

  - If fail → rollback to previous partition and report to server.

- LED: Blue or distinct blink pattern during OTA download; revert to prior pattern after complete.

- OTA safety: OTA will be blocked if heater is active or temperature above T_ota_block threshold — to avoid mid-operation bricking. Optionally allow OTA when machine is idle only.

- F. Diagnostics, Logging & Maintenance

- Local logs: circular log file of last N events includes temperature, knob changes, relay toggles, OTA events, faults. Stored in flash with wear-leveling.

- Remote diagnostics: on request, device can upload logs to server over secure channel (e.g., HTTPS or MQTT with TLS).

- Factory test mode: special sequence for production test (exercises heater, sensor, LED, Wi-Fi), automatically logs serial output for QA.

- G. Power OFF & Safe Shutdown

- On user switch OFF or mains removal:

  - If heater active → disable heater immediately, record event.

  - Flush last telemetry & logs to persistent storage (if powered).

  - If power removal imminent, use supercap energy to close files and store last state.

- On next boot → self-test will detect improper shutdown and raise a soft alert in logs.

- 3 — State Machine (compact)

| - State | - Entry Trigger | - Behavior | - Exit Trigger |
| --- | --- | --- | --- |
| - Power Off | - No power | - Heater off, LEDs off | - Power On |
| - Boot / Self-test | - Power On | - Short self tests; white blink | - Config required / Registered |
| - Configuration (AP) | - Unconfigured or reset | - Advertise unique AP, Orange blink | - Provision success / Timeout |
| - Network Connect | - Wi-Fi config present | - Connect to AP, validate subscription | - Connected / Provision failed |
| - Registered / Idle | - Subscription OK | - Green LED, accept commands | - Knob change / App start heating |
| - Heating | - T_current < T_set − H | - Relay ON, white/pulse LED | - T_current >= T_set + H |
| - Fault | - Sensor fail / Overtemp / Relay fault | - Heater OFF, red blink, notify cloud | - Manual reset (after fault cleared) |
| - OTA | - OTA Trigger (idle only) | - Download + verify + swap | - OTA success / rollback |

- 4 — Mobile App & Backend Interaction Summary

- App: login → add product (scan QR) → connect to device AP → send Wi-Fi creds → app receives confirmation of registration.

- Backend: authenticates user, validates subscription, records device-user mapping, authorizes OTA and remote commands.

- Command path options (choose with client):

  - Direct local: app connects to device via local network (LAN).

  - Cloud mediated: app → backend → device (MQTT or long polling).

  - Hybrid: both allowed; cloud used for remote management, local for faster provisioning.

- 5 — Edge & Error Cases (must be handled)

- Multiple devices nearby: unique SSID per device ensures correct selection. App must show device UID and verify scanned QR matches AP UID.

- Duplicate UID / Clone unit: device must authenticate via backend to detect duplicates; flag suspected clones.

- Intermittent Wi-Fi: device retries with exponential backoff; fall back to AP mode after M failed attempts.

- SD/Flash full or corrupt: rotate logs; if critical corruption, revert to factory settings (requires manual intervention).

- Unauthorized OTA: use signed firmware and auth tokens to block rogue updates.

- 6 — Telemetry & Alerts (recommended)

- Periodic telemetry (every 5–30 minutes): UID, T_current, T_set, heater_status, error_flags, RSSI.

- Immediate alerts (push/PUSH or webhook): sensor failure, overtemp, OTA failure, subscription expiry.

- 7 — Acceptance Criteria (for POC → move to PCB)

- POC will be considered successful when:

- Device boots and enters configuration AP when unprovisioned.

- Mobile app can connect via AP and push credentials; device connects to Wi-Fi.

- Device validates subscription successfully and turns Green.

- Knob and DS18B20 integrate and control heater reliably with defined hysteresis.

- OTA can be performed and rolled back safely.

- Multi-chip test passes on 3 different ESP32-C5 samples with identical behavior.