Nice — I inspected your uploaded sketch and extracted the exact GPIO pins the code is using. I’ll list them exactly as they appear in your code, map each to the hardware item you described, and give a few short, practical notes (wiring/pullups/safety) so you can prepare your board & wiring before flashing.

Exact GPIOs used (from your sketch)

- ONE_WIRE_BUS = PA27 → DS18B20 temperature sensor data (OneWire)

- LED2 = PA15 → indicator LED used to show shouldHeat state

- LED_STATUS = PA12 → status LED (blink, general status)

- transistorPin = PA26 → heater control (drives transistor / MOSFET; code uses digital HIGH/LOW)

- analogInPin = PB3 → knob / analog input (the potentiometer)

You can find these in your sketch as #define ONE_WIRE_BUS PA27, #define LED2 PA15, #define LED_STATUS PA12, #define transistorPin PA26, #define analogInPin PB3.

How the code uses them (behavioral summary)

- DS18B20 (PA27) — DallasTemperature library: sensors.requestTemperatures(), getTempCByIndex(0) etc. Expect a single-sensor setup on that pin.

- Knob (PB3) — read with analogRead(analogInPin), mapped to control thresholds / setpoints in the logic.

- Heater control (PA26) — simple digitalWrite(transistorPin, HIGH/LOW) (ON/OFF). No PWM in the current code — purely digital switching.

- LEDs (PA15 / PA12) — used for visual feedback: LED2 reflects heating state, LED_STATUS blinks / shows status.

Important wiring notes & checks

- DS18B20 wiring

  - DATA → PA27

  - VCC → 3.3 V

  - GND → GND

  - Add a pull-up resistor 4.7 kΩ between DATA and 3.3 V (required unless your sensor/module has one onboard).

- Knob / Potentiometer (analogInPin PB3)

  - Middle wiper → PB3

  - Other ends → 3.3 V and GND

  - Use 3.3 V reference (BW16 runs 3.3 V).

- Heater drive (transistorPin PA26)

  - If using an N-channel MOSFET to switch the heater on the low side: connect MOSFET gate to PA26 (via a small gate resistor ~100Ω), source to GND, drain to heater negative; heater positive → Vcc (or mains / appropriate supply depending on heater).

  - If switching mains (220 VAC) — DO NOT directly use an Arduino pin; require proper relay/module/SSR and mains safety isolation. The code currently toggles the pin digital HIGH/LOW (no mains safety code built-in).

- LEDs

  - Include a current-limiting resistor (e.g., 220Ω) when wiring LEDs to PA15 / PA12.

- Power rails & ground

  - Make sure module GND is common with sensor & adapter GND. Use stable 3.3 V supply (enough current).

Safety & reliability suggestions (important)

- If heater is mains-powered (220 VAC): use an SSR or properly rated relay and isolate control electronics. Add fuse and earth/ground protection. I can help design that wiring if needed.

- Add temperature safety cutoff (failsafe): hardware watchdog or secondary thermostat to avoid runaway heating.

- Debounce & filtering for analog knob (you probably already map & smooth, but consider adding small smoothing).

- Use logic-level MOSFET if switching DC heater (e.g., IRLZ44N-like alternative rated for your current); add gate resistor and pull-down to prevent floating gate.

- If inductive loads (motor, pump) use freewheel diode / snubber / TVS as appropriate.

Anything missing from your component list?

Based on the code and what you told me, the code expects these components only:

- DS18B20 (connected to PA27)

- 1 × indicator LED (PA15)

- 1 × status LED (PA12)

- Heater switching transistor/MOSFET driven from PA26

- Knob/potentiometer into PB3

You might consider adding these hardware items (if not already planned):

- 4.7 kΩ pull-up for DS18B20 data line (if not present on sensor board)

- Gate resistor and logic-level MOSFET (if switching DC heater) or SSR/relay + optoisolation (if switching mains)

- Fuse / thermal cutout for safety

- Decoupling capacitor on 3.3V rail for stability under load

If you want, I can now:

- Draw a small wiring diagram showing how to connect each component to the BW16 pins above (and include resistor values).

- Suggest exact MOSFET/SSR/transistor part numbers for your heater type (tell me: DC heater or AC/mains heater?).

- Add software improvements (e.g., PWM soft-start, hysteresis settings you might want exposed via the knob).

Tell me which of these you'd like next and I’ll produce the wiring diagram or component suggestions right away.