The client has two product for now. Generally we are calling them as Version-1 and Version-2.

Project description/Functional Flow:

The product is specifically developed for heating Wax on specific temperature and as soon as the equipment reaches at some thrashhold limit, it will cut off the heater.

Consider that product is new and we have unboxed it.

The product needs to be plugin to 110V plug. As soon as the equipment get switch-on on 110V, the LED_Light will start rotating on the system.

The equipment also has its internal memory (EEPROM), where it can keep some data. The Chip keeps 256 KB of data in ESP3266.

The equipment reads the data in the EEPROM and cross check if that is blank or it that has any data. If that is Blank/Empty, it publishes (Advertise) the WiFi name and gives an option to the user to connect the appropriate WiFi, along with the username and password. The entered username, password and the WiFi name gets saved into the internal memory (EEPROM) and gets connected to that network.

As soon as the internet/WiFi gets connected, the LED_Light starts and showing different color.

There is a knob attached to the equipment, that helps to increase/decrease the heater temperature. The heating module is attached with one thermostat that keeps on checking the temperature of the heater. As soon as the thermostat reaches to that thrash hold, it cut off the heater automatically.

Now, as soon as the customer switch on the equipment again, it performs the same procedure. Like, it reads the EEPROM, if it finds the WiFi username/password and if that gets connected.