Awesome — I loaded your MellaFirmware.ino and mapped the code to your product’s flow so you can see exactly “what in the code does what in the machine.” I also pointed out a couple of gaps vs your description.

I’ve put two quick views in your workspace:

- Functions overview (what functions exist + a short preview)

- Feature ↔ Code mapping (where each product feature lives in the code)

How the firmware implements your flow

- Power-on & LED behavior

- Code sets up an LED status pin and uses it for blink/status indications: pinMode(LED_STATUS, OUTPUT) and digitalWrite(LED_STATUS, ...) (lines ~120, ~198, ~200, ~1361–1363).

- There’s a call blink_led(...) (e.g., line 190), but its definition is likely in config.h (not in the uploaded folder). So LED “rotating” is represented by blinks right now, not an RGB animation.

- Internal memory (EEPROM)

- ESP8266 doesn’t have real EEPROM; it emulates it in flash. Your code allocates 512 bytes with EEPROM.begin(512); (line 117) and uses EEPROM.read/write/commit throughout (e.g., lines 158, 449–461, 540+).

- So it’s not “256 KB of EEPROM”; it’s 512 bytes currently (configurable by EEPROM.begin(N)).

- First-time Wi-Fi provisioning

- The sketch uses ESP8266 SmartConfig, not an AP/captive-portal flow:

  - WiFi.beginSmartConfig(); and checks WiFi.smartConfigDone() (lines ~201, 209, 215).

  - That means you push SSID/password from a phone app (ESP-Touch/AirKiss), rather than the device “advertising a Wi-Fi network” for you to join.

- Credentials and other config data are persisted via EEPROM (see save_to_eeprom(...), read_eeprom() around lines 540–590).

- If you want the “device advertises its own Wi-Fi, user joins and enters credentials” flow, consider replacing SmartConfig with WiFiManager (SoftAP + captive portal). The code doesn’t include WiFiManager today.

- On Wi-Fi/Internet connect → LED changes

- The code sets LED pin and calls blink on various network/MQTT events (e.g., lines 190, 475, 480, 1361–1363). You can map “connected” vs “trying” states to specific blink patterns or colors (if RGB) inside blink_led().

- Temperature sensing (thermostat behavior)

- Uses DallasTemperature + OneWire with a DS18B20-style digital sensor:

  - OneWire oneWire(ONE_WIRE_BUS); and DallasTemperature sensors(&oneWire);

  - Reads temp_c after sensors.requestTemperatures() (lines ~270–278).

- This is a sensor, not a mechanical thermostat. The thermostat logic (cutoff) is implemented in code by comparing temp_c to thresholds (see below).

- Knob → Temperature setpoint

- Knob on A0 (analog): #define analogInPin A0 → sensorValue = analogRead(analogInPin) (lines ~250–257).

- Mapped to 10 steps: mappedOutput = map(..., 0..25, 0..10) then handled in set_temp(int knob_input) (starting ~1096).

- Inside set_temp(...) you publish debug and branch via switch(knob_input) to decide the MOSFET/relay control with a ladder of cutoff temperatures, e.g.:

  - if (temp_c <= 30 + calibration_offset) ...

  - <= 36 + calibration_offset, <= 42 + 5, <= 48, … up to <= 90, <= default_temp

  - This is your software thermostat: heater on below threshold, off above.

- Heater drive & safety cutoff

- Heater is controlled via RELAY (and MOSFET patterns): pinMode(RELAY, OUTPUT) (line 119), digitalRead(RELAY) checks and status writes later.

- A series of “Mosfet Case X ON/OFF” branches (lines ~1117–1314) drive pins chrisPin1=12, chrisPin2=13, transistorPin=14 depending on knob/temperature state.

- The “cut-off” is achieved by the if-chains against temp_c levels (above).

- Cloud/telemetry (MQTT)

- Uses MQTT server + creds: client.setServer(...), client.connect(...) (lines ~286–296, ~514–517).

- Topics/subscriptions occur in setupMQTT(); publishing status via updateMqtt(int status) (from line ~1019). You also publish knob values and debug strings at a few points.

- This supports remote status/commands and helps QA deployments for V1/V2.

- Schedules & last state in EEPROM

- There’s a compact, colon-delimited schedule string persisted in EEPROM; parser slices out per-day start/end hours/minutes (lines ~540–590).

- Last power state is saved at SAVED_STATUS_ADDRESS (lines ~449–461). This is what lets you restore behavior on reboot.

- Ticker

- Ticker tasker; is declared (line 18) but I don’t see any tasker.attach(...) calls; it’s currently unused. If you planned periodic jobs (LED animation, MQTT keepalive, temp polling), you can add it back.

Where this diverges (a bit) from your prose

- “Device advertises Wi-Fi”: current code uses SmartConfig (no AP). If you truly want the device to broadcast e.g. Mella-Setup and open a portal, switch to WiFiManager (SoftAP) and save SSID/password to EEPROM/flash.

- “EEPROM 256 KB”: ESP8266 emulates EEPROM in flash. You’ve set EEPROM.begin(512), so you have 512 bytes available (not 256 KB). You can increase that number, but keep in mind it shares with flash wear.

- LED “rotating color”: the code toggles a single LED_STATUS pin. If your hardware has an RGB/Neopixel ring, you’ll need the appropriate library (e.g., Adafruit_NeoPixel or FastLED) and patterns (rotate/rainbow) — I don’t see those includes yet.

Quick compile fixes (based on your earlier errors)

Add missing headers at the top (they’re used but not included):

#include <PubSubClient.h>

#include <Ticker.h>

#include <EEPROM.h>

(You already include ESP8266WiFi.h, ESP8266WiFiMulti.h, OneWire.h, DallasTemperature.h, ESP8266httpUpdate.h.)

Suggestions to make V1/V2 crystal-clear

- Provisioning mode

  - V1: keep SmartConfig (fast setup via app).

  - V2: add WiFiManager SoftAP fallback after, say, 60s of failed SmartConfig. Show different LED pattern while provisioning.

- LED state machine

  - Map distinct states → patterns:
Boot (slow blink), Provisioning (double blink), Wi-Fi OK (solid), MQTT OK (short pulse every 5s), Heating (fast blink), Overtemp/fault (SOS).

- Thermostat logic

  - Centralize target setpoint from knob into a single variable, apply hysteresis (e.g., ±2 °C) to avoid relay chattering.

  - Add a hard safety: if DEVICE_DISCONNECTED_C or temp_c implausible, force heater OFF and flash fault code.

- Persistence

  - Keep using EEPROM emulation, but wrap with a struct (versioned) and store checksum so partial writes can be detected.

- Diagnostics

  - Continue MQTT publishes for: firmware version, Wi-Fi RSSI, current temp, setpoint, heater state, last fault.

- Ticker usage

  - Use Ticker for periodic tasks: temp sample (e.g., 1 s), MQTT heartbeat (10 s), LED animation (50–100 ms), saving state (debounced every 5 s when changed).

If you want, I can:

- convert first-time setup to SoftAP + captive portal,

- add a small hysteresis thermostat with a single setpoint,

- and implement a clean LED state machine that matches your “rotating color” spec.